Ivi com driver examples.

The examples found in this section were installed in the ivi\drivers\kepco\examples folder during installation of the IVI com driver.  Iin this section the full text of the example is not present.

Output / measurement example:

STEVE (Full code snippet is: C output example)

All programs written in C++ must open the driver and have some standard H files.

/*~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

This program demonstrates how to set a voltage and current

measure the output voltage

using the IVI-COM driver.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~*/

#include <stdafx.h>

#include <stdio.h>

#include <conio.h>

#include <atlbase.h>

#import "IviDriverTypeLib.dll" no_namespace

#import "IviDCPwrTypeLib.dll" no_namespace

#import "KepcDCPwr.dll" no_namespace

The driver dlls must be loaded and initialized to operate:

hr = CoInitialize(NULL);

if (FAILED(hr))

exit(1);

// create a safe pointer for interface access

IKlpPtr driverPtr=NULL;

hr = driverPtr.CreateInstance(__uuidof(KepcoDCPwr));

if (FAILED(hr))

exit(1);

Once initialized communication with the KLP must be established.  This is an example of opening the driver at GPIB address 6.  The id query has been set true.  Failure to set this true will reduce the functionality of the driver to the DCPWR class capabilities.  This prevents functions such as setting the virtual model, List and other advanced features of the KLP from being accessed.

// open the instrument for communication

hr = driverPtr->Initialize(

"GPIB0::6::INSTR",  //Visa address,(not applicable if simulation=true)

VARIANT_TRUE,    // ID query

VARIANT_TRUE,     // Reset

LPCTSTR("Cache=true, InterchangeCheck=false,    QueryInstrStatus=true,  Simulate=false")); //IVI options

The driver is a linked list of pointers to properties and functions. To read a property, all is required is to locate the porpoerty .  This is an example of reading the firmware revision of the KLP.  To read this property the identification must have been performed..

bstrInstrFwRev = driverPtr->Identity->InstrumentFirmwareRevision;

As can be seen the accessing of the properties requires many pointer operations.. Our examples though use pointer redirection to make the operations a little clearer.

// get pointers to the needed interfaces

IKlpOutputPtr outputPtr;

outputPtr = driverPtr->Outputs->GetItem(OLESTR("Output"));

IKlpMeasurementPtr measurementPtr;

measurementPtr = driverPtr->Measurements- >GetItem(OLESTR("Measurement"));

The setting of  voltage and current is then very simple.

// set voltage

hr = outputPtr->VoltageLevel(75.0);

if (FAILED(hr))

exit(1);

// set the current limit

hr = outputPtr->CurrentLimit(10);

if (FAILED(hr))

exit(1);

// enable the output

outputPtr->Enabled = true;

Sleep(500);

The getting of a measurement is accessing not a variable but a function.

// measure the voltage

double measVoltage;

measVoltage = measurementPtr->Measure(KepcoDCPwrMeasurementVoltage);

List example:

The list example is written in C#.

STEVE file snippet is C LIST EXAMPLE

C# uses com drivers the easiest of all environments.

To initialize the driver is easy in that the only the driver name is required.

using System;

using KepcoDCPwr.Interop;

// Create driver instance.

KepcoDCPwr KLPdriver;

KepcoDCPwr.Interop.Output outputPtr;

KepcoDCPwr.Interop.IList  listPtr;

KLPdriver = new KepcoDCPwrClass();

The connection between the host and the KLP is then established by the code:

KLPdriver.Initialize("GPIB0::6::INSTR",

true,

true,

"Cache=true,InterchangeCheck=false,QueryInstrStatus=true,Simulate=false");//Optional ivi options

// get references to the needed interfaces

outputPtr = KLPdriver.Outputs.get_Item(driver.Outputs)	;

listPtr = KLPdriver.List.get_Item(driver.Lists);

The list functionality uses arrays so the arrays must be created and the code creates these arrays and then fills them with data.

//create arrays for the ListPoints method

double[] voltList = new double [6];

double[] currList = new double [6];

double[] dwellTime = new double [6];

voltList[0] = 10;

voltList[1] = 20;

voltList[2] = 30;

voltList[3] = 40;

voltList[4] = 50;

voltList[5] = 60;

currList[0] = 2;  // select max current for each step

currList[1] = 2;

currList[2] = 2;

currList[3] = 2;

currList[4] = 2;

currList[5] = 2;

dwellTime[0] = 1;

dwellTime[1] = 2;

dwellTime[2] = 3;

dwellTime[3] = 4;

dwellTime[4] = 5;

dwellTime[5] = 0.01;

To send the lists to the KLP with the IVI com driver is performed by the command   Note the arrays are passed by reference to the driver.

listPtr.ListPoints(ref voltList, ref currList, ref dwellTime);

The list then should be set to the number of executions and the list started:

// Set the number of executions.

listPtr.count = 2;

// initiate

outputPtr.listInitiate();

The driver should be closed at the end using the follwing code.  The conditional allows the close to be added to the end of the program without getting any runtime erros.

//' Close driver if initialized.

if (KLPdriver.Initialized == true ) KLPdriver.Close();

visual Basic

Visual Basic has its own methods to support the civi.com driver.

Public driver As KepcoDCPwr

Public outputPtr As IKLPOutput

Public protectionPtr As IKLPProtection

Public measurementPtr As IKLPMeasurement

Set driver = New KepcoDCPwr

Once the driver has been   initialized, the connection between the host and the KLP is established using a stantard visa resource string.    It is important that the initialize function be passed a true as its second argument to allow the KLP specific features to be enabled.  In this example, no KLP feature such as list is required.

' initialize the driver - note ID device is true

driver.Initialize "GPIB0::6::INSTR", _

True, _

True, _

"Cache=true,InterchangeCheck=false,QueryInstrStatus=true,Simulate=false"

Dim result As Boolean

result = driver.Initialized

Set outputPtr = driver.Outputs.Item(driver.Outputs)

Set protectionPtr = driver.Protections.Item(driver.Protections)

Set measurementPtr = driver.Measurements.Item(driver.Measurements)

Using the pointers it becomes very easy to set the voltage and currnet levels..  Each of the parameters is just loaded with the value to be set using individual statelments.

' set voltage level

outputPtr.VoltageLevel 75

' enable OV protection and set OV level

protectionPtr.ConfigureOVP 15

' set current level

outputPtr.CurrentLimit 10

' enable the output

outputPtr.Enabled = True

Measuring the output is performed by accessing a function and not a parameter.

' Measure the voltage

Dim measVoltage As Double

measVoltage = measurementPtr.Measure(KepcoDCPwrMeasurementVoltage)

The closing of the driver is the last step in any program:

driver.Close