Kepco BOP488 Power Supply

________________________________________________________________________________

Introduction:

This instrument module provides GPIB programming support for the

Kepco BOP488 Power Supply.  It contains functions for opening,

configuring, setting output, and closing the instrument via the GPIB

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

- the instrument is connected to the GPIB.

- the GPIB address supplied to the initialize function must match

the GPIB address of the instrument.

- there is one unused device name in the handler

________________________________________________________________________________

LabWindows Libraries:

In order to use this instrument module outside of the LabWindows envi-

ronment, it must be linked with the following LabWindows libraries:

- Formatting and I/O

- GPIB

________________________________________________________________________________

Error Codes:

All error codes are returned as an integer

A program should examine this value after each function call is made

to determine if any errors occurred

________________________________________________________________________________

Function Tree Layout:

Kepco BOP488 Power Supply

Initialize

Configure

Set Output

Close

________________________________________________________________________________

Function           : Initialize

QuickBASIC Syntax  : Error% = bop488.init(Address%)

C Syntax           : int bop488_init(Address);

int Address;

Purpose            :  Panel Name:    Initialize

Description:   opens the instrument

and sets its GPIB address

to the specified value

Parameter List     : Address - Integer

Control Name:  Address

Description:   Specifies the address on the

GPIB to be used to communicate

with the BOP unit.  This value

must correspond to the settings

of the address DIP switches on the

BOP.

Variable Type: Integer

Valid Range:   0 to 30

Return Value       :  Control Name:  Error

Description:   Displays status relating to the

function call.

Variable Type: Integer

Returned Values:

0   Success

-1   GPIB Address out of range

220   Unable to open instrument

223   Device at specified address is not

the correct instrument

230   GPIB write error

233   Unable to configure GPIB address

QuickBASIC Example : Error% = bop488.init(??)

C Example          : int bop488_init(??);

________________________________________________________________________________

Function           : Configure

QuickBASIC Syntax  : Error% = bop488.config(Mode%, Range%, Card.Type%, _

Maximum.Voltage#, Maximum.Current#)

C Syntax           : int bop488_config(Mode, Range, Card_Type,

Maximum_Voltage, Maximum_Current);

int Mode;

int Range;

int Card_Type;

double Maximum_Voltage;

double Maximum_Current;

Purpose            :  Panel Name :   Configure

Description :  Sets up the programming mode of

the BOP.

i.e. VOLT,CURR;HIGH,LOW

Parameter List     : Mode - Integer

0 - VOLTAGE (Default)

1 - CURRENT

Range - Integer

0 - +10 (Default)

1 - -10

2 - +1

3 - -1

Card Type - Integer

1 - 488-D

0 - 488-B (Default)

Maximum Voltage - Real

Control Name:  Maximum Voltage

Description:   Sets the full scale voltage value

of the BOP. This value should be

set to the maximum rated voltage

value of your unit.

Variable Type: Real

Valid Range:   0 - 10000

Maximum Current - Real

Control Name:  Maximum Current

Description:   Sets the full scale current value

of the BOP. This value should be

set to the maximum rated current

value of your unit.

Variable Type: Real

Valid Range:   0 - 1000

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Returned Values:

0    Success

-1    Invalid Mode value

-2    Invalid Range value

-3    Invalid Type value

-4    Invalid Voltage value

-5    Invalid Current value

230    GPIB write error

232    Instrument not initialized

QuickBASIC Example : Error% = bop488.config(??, ??, ??, ??, ??)

C Example          : int bop488_config(??, ??, ??, ??, ??);

________________________________________________________________________________

Function           : Set Output

QuickBASIC Syntax  : Error% = bop488.set(Voltage#, Current#)

C Syntax           : int bop488_set(Voltage, Current);

double Voltage;

double Current;

Purpose            :  Panel Name :   Set Output

Description :  Programs the output voltage and

current of the BOP.

Parameter List     : Voltage - Real

Control name:  Voltage

Description:   Selects the value of voltage to be

programmed.  The full scale value

must have been previously set by

the configure function, otherwise

a value of 10 will correspond to

full scale.

Variable Type : Real

Valid Range is set by the configure function

Current - Real

Control name:  Current

Description:   Selects the value of current to be

programmed.  The full scale value

must have been previously set by

the configure function, otherwise

a value of 10 will correspond to

full scale.

Variable Type : Real

Valid Range is set by the configure function

Return Value       :  Control name:   Error

Description:   Displays status relating to the

function call.

Returned Values:

0   Success

-1   Voltage value out of range

-2   Current value out of range

230   GPIB write error

232   Instrument not initialized

QuickBASIC Example : Error% = bop488.set(??, ??)

C Example          : int bop488_set(??, ??);

________________________________________________________________________________

Function           : Close

QuickBASIC Syntax  : Error% = bop488.close

C Syntax           : int bop488_close();

Purpose            :  Panel Name:    Close

Description:   Takes the instrument off-line.

Note: The  instrument must  be reinitialized

to use it again.

Return Value       :  Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Returned Values:

0   Success

221   Unable to close instrument

232   Instrument not initialized

QuickBASIC Example : Error% = bop488.close

C Example          : int bop488_close();