Kepco SNR488 Power Supply Controller

________________________________________________________________________________

Introduction:

This instrument module provides GPIB programming support for the

Kepco SNR488 Power Supply Controller.  It contains functions

for opening, configuring, taking measurements from, and closing

the instrument via the GPIB

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

- the instrument is connect to the GPIB.

- the GPIB address supplied to the initialize function must match

the GPIB address of the instrument.

- there is one unused device name in the handler

________________________________________________________________________________

LabWindows Libraries:

In order to use this instrument module outside of the LabWindows envi-

ronment, it must be linked with the following LabWindows libraries:

- Formatting and I/O

- GPIB

________________________________________________________________________________

Error Codes:

All error codes are returned as the return value of each function.

A program should examine this value after each function call is made

to determine if any errors occurred

________________________________________________________________________________

Function Tree Layout:

Kepco SNR488 Power Supply Controller

Initialize

Configure

Set Output

Close

________________________________________________________________________________

Function           : Initialize

QuickBASIC Syntax  : Error% = snr488.init(Address%)

C Syntax           : int snr488_init(Address);

int Address;

Purpose            : Opens the instrument and sets its GPIB address to the

specified value

Parameter List     : Address - Integer

Description:   Specifies the address on the

GPIB to be used to communicate

with the instrument.  This value

must correspond to the current

internal setting of the

instrument.

Valid Range:   0 to 30

Error Codes        :         0   Success

-1   GPIB Address out of range

220   Unable to open instrument

223   Device at specified address is not

the correct instrument

230   GPIB write error

233   Unable to configure GPIB address

________________________________________________________________________________

Function           : Configure

QuickBASIC Syntax  : Error% = snr488.config(Channel%, Range%, Card.Type%, _

Maximum.Voltage#, Maximum.Current#)

C Syntax           : int snr488_config(Channel, Range, Card_Type,

Maximum_Voltage, Maximum_Current);

int Channel;

int Range;

int Card_Type;

double Maximum_Voltage;

double Maximum_Current;

Purpose            : Sets up a programming card in one of the slots of the

SNR488 to be used for a particular model power supply.

Parameter List     : Channel - Integer

0 - 0 (Default)

1 - 1

2 - 2

3 - 3

4 - 4

5 - 5

6 - 6

7 - 7

Range - Integer

0 - 0 (Default)

1 - 1

2 - 2

3 - 3

4 - 4

5 - 5

6 - 6

7 - 7

8 - 8

9 - 9

10 - A

11 - B

12 - C

13 - D

14 - E

15 - F

Card Type - Integer

1 - 488-D

0 - 488-B (Default)

Maximum Voltage - Real

Description:   Sets the full scale voltage value

of the program card for the

selected channel #.  This value

should be set to the maximum rated

voltage value of the power supply

that will be programmed by that

card.

Valid Range:   0 - 10000

Maximum Current - Real

Description:   Sets the full scale current value

of the program card for the

selected channel #. This value

should be set to the maximum rated

current value of the power supply

that will be programmed by that

card.

Valid Range:   0 - 1000

Error Codes        :         0   Success

-1   Channel out of range

-2   Voltage value out of range

-3   Current value out of range

230   GPIB write error

232   Instrument not initialized

________________________________________________________________________________

Function           : Set Output

QuickBASIC Syntax  : Error% = snr488.set(Channel%, Voltage#, Current#)

C Syntax           : int snr488_set(Channel, Voltage, Current);

int Channel;

double Voltage;

double Current;

Purpose            : Programs the output voltage and current of a selected

power supply.

Parameter List     : Channel - Integer

0 - 0 (Default)

1 - 1

2 - 2

3 - 3

4 - 4

5 - 5

6 - 6

7 - 7

Voltage - Real

Description:   Selects the value of voltage to be

programmed.  The full scale value

must have been previously set by

the configure function, otherwise

a value of 10 will correspond to

full scale.

Valid Range is set by the configure function

Current - Real

Description:   Selects the value of current to be

programmed.  The full scale value

must have been previously set by

the configure function, otherwise

a value of 10 will correspond to

full scale.

Valid Range is set by the configure function

Error Codes        :         0   Success

-1   Channel out of range

-2   Voltage value out of range

-3   Current value out of range

230   GPIB write error

232   Instrument not initialized

________________________________________________________________________________

Function           : Close

QuickBASIC Syntax  : Error% = snr488.close

C Syntax           : int snr488_close();

Purpose            : Takes the instrument off-line.

Note: The  instrument must  be reinitialized

to use it again.

Error Codes        :         0   Success

221   Unable to close instrument

232   Instrument not initialized