Kepco TMA488-27 Power Supply Controller

____________________________________________________________________________

Introduction:

This instrument driver provides programming support for the following

Kepco Power Supply Controllers:

TMA488-27

TMA 4882-27

MST 488-27

MBT-G

It contains functions for opening, configuring, taking measurements from,

and closing the instrument.

____________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match

the GPIB address of the instrument.

-  there must be at least one unused device name in the handler.

____________________________________________________________________________

LabWindows Libraries:

In order to use this instrument driver outside of the LabWindows

environment, it must be linked with the following LabWindows libraries:

-  GPIB Library

-  Format and I/O Library

-  LabWindows System Library

National Instruments strongly recommends that you use the LWMAKE utility to

build any standalone executable program based on this instrument driver.

____________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an

instrument driver function to determine if any error occurred. Possible

error codes and their meanings are listed with the corresponding instrument

driver function.

____________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the Kepco TMA488-27 Power Supply Controller

instrument driver. The functions are listed in alphabetical order with

a description of the function, C and BASIC syntax of the function,

a description of each parameter, and possible error codes.

____________________________________________________________________________

Function Tree Layout:

Kepco TMA488-27 Power Supply Controller     Function Name:

Initialize                               tma488_init

Set Output Level                         tma488_set

Set Output On/Off                        tma488_output

Read Output Level                        tma488_read

Reset a Channel                          tma488_reset

Read Status                              tma488_status

Low Level I/O

Send Command                          tma488_cmnd

Read Input                            tma488_rec_str

Close                                    tma488_close

____________________________________________________________________________

Instrument         : Kepco TMA488-27 Power Supply Controller

Instrument Name: Kepco TMA488-27 Power Supply

Controller

Description:     This instrument module provides

programming support for the

Kepco TMA488-27, TMA 4882-27,

MST 488-27, & MBT-G.  The module

is divided into the following

functions:

Functions/Classes:

(1)  Initialize:

Initializes the power supply

(2)  Set Output Level:

Sets the voltage and current

(3)  Set Output On/Off:

Sets output on/off via the output relay

(4)  Read Output Level:

Reads back a voltage or current

(5)  Reset a Channel:

Resets the selected channel

(6)  Read Status:

Reads back status info. from the TMA488

(7)  Low Level I/O: (Class)

This class contains functions for writing

and reading strings from the isntrument.

(8)  Close:

This function takes the instrument off the

GPIB bus and returns it to local mode.

____________________________________________________________________________

Following functions are in alphabetical order.

____________________________________________________________________________

Function Name      : tma488_close

C Syntax           : int tma488_close();

BASIC Syntax       : Error% = tma488.close

Purpose            : Takes the instrument off-line.

Note: The instrument must be reinitialized to use

it again.

Error Codes        :         0   Success

221   Error closing instrument

232   Instrument not initialized

____________________________________________________________________________

Function Name      : tma488_cmnd

C Syntax           : int tma488_cmnd(Command);

char *Command;

BASIC Syntax       : Error% = tma488.cmnd(Command$)

Purpose            : Allows direct entry of a CIIL command to the

instrument.

See the instrument's manual for a complete description

of CIIL commands

Parameter List     : Command

Description:   Any CIIL command described in

the instrument user's manual.

Variable Type: Character String (char[])

Default Value: "CIIL\n"

Error Codes        :         0   Success

230   GPIB write error

232   Instrument not initialized

____________________________________________________________________________

Function Name      : tma488_init

C Syntax           : int tma488_init(GPIB_Address);

int GPIB_Address;

BASIC Syntax       : Error% = tma488.init(GPIB.Address%)

Purpose            : Opens the instrument and sets its GPIB address to the

specified value

Parameter List     : GPIB Address

Description:   Specifies the address on the

GPIB to be used to communicate

with the instrument.  This value

must correspond to the current

internal setting of the

instrument.

Variable Type: Short Integer

Valid Range:   0 to 30

Default Value: 6

Error Codes        :         0   Success

-1   GPIB Address out of range

220   Unable to open instrument

223   No device at the specified address

230   GPIB write error

233   Unable to configure GPIB address

____________________________________________________________________________

Function Name      : tma488_output

C Syntax           : int tma488_output(Channel, Output);

int Channel;

int Output;

BASIC Syntax       : Error% = tma488.output(Channel%, Output%)

Purpose            : Enables or disables a power supply's output.

Parameter List     : Channel

Description:   Selects the power supply to be

programmed. The channel number

corresponds to the DIP switch

settings on the power supply.

Variable Type: Short Integer

Valid Range:   0 - 30

Default Value: 1

Output

Description:   Selects whether the output

is enabled or disabled

Variable Type: Short Integer

Valid Range:   0 - Disabled (Default)

1 - Enabled

Error Codes        :         0   Success

-1   Channel number out of range

-2   Output Mode out of range

230   GPIB write error

232   Instrument not initialized

____________________________________________________________________________

Function Name      : tma488_read

C Syntax           : int tma488_read(Channel, Mode, Value);

int Channel;

int Mode;

double *Value;

BASIC Syntax       : Error% = tma488.read(Channel%, Mode%, Value#)

Purpose            : Reads back the voltage or current of a power supply

Parameter List     : Channel

Description:   Selects the power supply to be

read. The channel number

corresponds to the DIP switch

settings on the power supply.

Variable Type: Short Integer

Valid Range:   0 - 30

Default Value: 1

Mode

Description:   Selects which parameter to read

Variable Type: Short Integer

Valid Range:   0 - Voltage (Default)

1 - Current

Value

Description:    Returns the value read.

Variable Type:  Double *

Error Codes        :         0   Success

-1   Channel number out of range

-2   Mode out of range

230   GPIB write error

231   GPIB read error

232   Instrument not initialized

236   Error interpreting instrument response

____________________________________________________________________________

Function Name      : tma488_rec_str

C Syntax           : int tma488_rec_str(Input);

char *Input;

BASIC Syntax       : Error% = tma488.rec.str(Input$)

Purpose            : This function must be preceeded by a query command

such as "STA" or "FTH VOLT", sent via the tma488_cmnd

function.

Note: This function is normally not needed,but is

provided as a lower level function to be used in

conjunction with the tma488_cmnd function.

Parameter List     : Input

Description:   Returns the string read from the

instrument.

Variable Type: String at least 40 long (char[40])

Error Codes        :         0   Success

231   GPIB read error

232   Instrument not initialized

____________________________________________________________________________

Function Name      : tma488_reset

C Syntax           : int tma488_reset(Channel);

int Channel;

BASIC Syntax       : Error% = tma488.reset(Channel%)

Purpose            : Resets a power supply to its power on state

Parameter List     : Channel

Description:   Selects the power supply to be

programmed. The channel number

corresponds to the DIP switch

settings on the power supply.

Variable Type: Short Integer

Valid Range:   0 - 30

Default Value: 1

Error Codes        :         0   Success

-1   Channel Number out of range

230   GPIB write error

232   Instrument not initialized

____________________________________________________________________________

Function Name      : tma488_set

C Syntax           : int tma488_set(Channel, Mode, Voltage, Current);

int Channel;

int Mode;

double Voltage;

double Current;

BASIC Syntax       : Error% = tma488.set(Channel%, Mode%, Voltage#, Current#)

Purpose            : Programs the output voltage and current of a selected

power supply.

Parameter List     : Channel

Description:   Selects the power supply to be

programmed. The channel number

corresponds to the DIP switch

settings on the power supply.

Variable Type: Short Integer

Valid Range:   0 - 30

Default Value: 1

Mode

Description:   Selects the output parameter

to be regulated.

Variable Type: Short Integer

Valid Range:   0 - Constant Voltage (Default)

1 - Constant Current

Voltage

Description:   Selects the value of voltage

to be programmed.

Variable Type: Double

Default Value: 0

Current

Description:   Selects the value of current

to be programmed.

Variable Type: Double

Default Value: 0

Error Codes        :         0   Success

-1   Channel Number out of range

-2   Mode out of range

230   GPIB write error

232   Instrument not initialized

____________________________________________________________________________

Function Name      : tma488_status

C Syntax           : int tma488_status(Status);

char *Status;

BASIC Syntax       : Error% = tma488.status(Status$)

Purpose            : Reads back status information from the instrument

Parameter List     : Status

Description:   Returns the status string.

Variable Type: String at least 40 long (char[40])

Error Codes        :         0   Success

230   GPIB write error

231   GPIB read error

232   Instrument not initialized